Upgrade VT-OS OS

- Hard reset the Helio by pressing the RESET button and ON button for about 1 second, release the ON button and then RESET button. The Helio is ready to enter the boot loader now.

- Enter the boot loader by holding the UP button and then ON button for 3 to 4 seconds. Release the ON button, then release UP button. Finally, DOWN button should be pressed. You should see the Helio show “Boot Loader” on the display.

- Place the Helio in the cradle and make sure that the cradle is connected to the PC.

- Type “DT 1 PDA.LZO 0x10000” if you are connecting the cradle to COM1 or  “DT 2 PDA.LZO 0x10000” for COM2. Press [ENTER] to start the system update process.

- Waiting about 2 to 3 minutes or until you get the message “Write Check Complete !”, if it takes longer that 7 minutes, press [ENTER] to stop the update and restart from step 2.

- Done. After Hard reset the Helio (see step 3), you can check the OS version form the System Information in the System Setup application.

* 	Note: the serial port may not act normally in DOS prompt under WIN95/98, try boot to DOS prompt directly if problems occur.