How to upgrade with an SD card

Step 1) format a 2gb or under SD card Fat32 then take and transfer  the NBO and CRC file of the rom you want to install to the sd card and insert the sd card into the axim. **do not try to install a rom that was not made for the exact model you are trying to upgrade it will not work**

Step 2) Press and hold the Power Button + WiFi (Hardware) Button

Step 3) While holding those two buttons, QUICKLY PRESS AND RELEASE the Reset Button on the back of your Axim. The "press and release" is for all practical purposes a simple "tap" of the reset button. NO NEED TO HOLD THE RESET BUTTON IN FOR ANY LENGTH OF TIME. NO NEED TO PRESS THE RESET BUTTON MULTIPLE TIMES.

Step 4) Continue to hold the Power + WiFi buttons until you see the white screen with a blue "banner" at the top which states the SD Image Loader version

Step 5) Once you see the above screen, release all buttons and the ROM image from your SD card should be listed on the screen shortly

Step 6) Press the Center Button (this is what is referred to as the "Action Button" or "Action Key") on the D-Pad (front directional pad on your Axim) TWICE QUICKLY

Step 7) Sit back and wait for your device to flash with the ROM

Windows mobile 5 files download then unrar and put the 2 files inside the folder on the sd card that is formatted fat 32

Windows mobile 6 files download then unrar and put the 2 files inside the folder on the sd card that is formatted fat 32