README for iPaq H36xx Option Pack Pro-Engineer files and analysis.

Please note from the ’Mechanical overview’ section of the Option Pack Specification:

All functional option packs must consist of four basic structures, including:

- a uniquely-curved, injection-molded plastic BASE part which enables any option pack to physically slide up and onto the back of any main unit, [see Pro-E part file: jc_jacket.prt ],

- a common universal connector [see Pro-E part file: foxconn-fx3.prt], soldered to a PCB with absolute dimensions as to the position of the connector and as to the thickness of the board but with less criteria as to the board area dimensions,

- an unknown volume of ‘new’ electronics that will, at minimum, communicate through the PCB and the universal connector, and

- a new injection-molded plastic top COVER part of specified shape at its bottom mating edge but of unspecified volume and shape (and, possibly, with unspecified openings) throughout its remainder. [See example Pro-E file: jc_cover_pcmcia_final.prt]

# Also, to assist the developer of new iPaq Option Packs, all Pro-E mechanical files for current PCMCIA and Compact Flash Option Packs (Jackets) have been made available as examples and for analysis.

# Please note the following instructions for various Pro-E Layers, Part files, Assemblies.

1:IF YOU OPEN THE ALL-INCLUSIVE ASSEMBLY FILE (CALLED jc_option_jackets.asm) FROM CD DISK OR ELSEWHERE, ALL THE PCMCIA ASSY LAYERS WILL BE THE DEFAULT STATUS.

2:IF YOU WANT TO VIEW AN EXAMPLE OF A COMPACT FLASH JACKET ASSY STATUS, PLEASE OPEN THE FOLLOWING LAYERS:

1: CF_COVER_FINAL             2: CF_PCB

3: CF_BOSS_F30                	  4: FINAL_SPRING

5: INSERT_36X42               	  6: JC_BUTTON

7: OP_JACK_0420             	  8: OP_JACK_F15

9: SCREW_COMPAQ             10: SCREW_JACK

3:IF YOU WANT TO VIEW AN EXAMPLE OF A PCMCIA_JACKET ASSY STATUS, PLEASE OPEN THE FOLLOWING LAYERS.

1: FINAL_SPRING       	          2: FLIP_SPRING

3: INSERT_36X42 		  	     	 4: JC_BUTTON

5: LENS                          	 6:OPJACK_0420

7: OP_JACK_F15                     8:PCMCIA_BOSS_F15

9: PCMCIA_BOSS_F30               10:PCMCIA_COVER_FINAL

11: PCMCIA_EJECT_ BUTTON        12:PCMCIA_FLIP

13: PCMCIA_PCB                    14:SCREW_COMPAQ

15: SCREW_JACK

4:IF YOU WANT TO VIEW THE CF PCB ASSEMBLY ONLY, YOU CAN OPEN THE FILE (pcb_assy_cf_lower.asm).

5:IF YOU WANT TO SEE THE PCMCIA PCB ASSY ONLY, YOU CAN OPEN THE FILE

(pcb_assy_pcmcia.asm).